Abstract ID: {to be filled by the organizers}

# Calibration of Organic Liquid Scintillator EJ-315 and Unfolding of Gamma Spectra

Primary author: Trinayan Saikia*1

Co-author: Sonika1, V. Singh2, L. Singh2

Presenter: Trinayan Saikia

Track Classification: Track 03

1 Department of Physics, Rajiv Gandhi University

2 Department of Physics, Central University of South Bihar

*Corresponding Author: trinayan.saikia@rgu.ac.in

This work presents the methodology and results of calibrating an organic liquid scintillator (EJ-315) [1] for gamma-ray spectroscopy using standard gamma sources, followed by spectral unfolding using a pre-determined detector response matrix using GEANT4 [2]. Energy calibration was performed via identification of known gamma peaks, and their Compton edge positions from Ba-133, Co-60, Na-22, and Cs-137 sources. Mathematical modelling of the detector’s linear light yield response was implemented. The calibrated spectra were unfolded using the GRAVEL [3] iterative technique to reconstruct the incident photon energy distribution.

References:

[1] C. C. Lawrence, A. Enqvist, M. Flaska, S. A. Pozzi, A. M. Howard, J. J. Kolata and F. D. Becchetti, Response characterization for an EJ315 deuterated organic-liquid scintillation detector for neutron spectroscopy, Nucl Instrum Methods Phys Res A 727, Elsevier, (2013) 21.

[2] J. Allison, K. Amako, J. Apostolakis, H. Araujo, P. A. Dubois, M. Asai, G. Barrand, R. Capra, S. Chauvie, et al., Geant4 developments and applications, IEEE Trans Nucl Sci 53, (2006) 270.

[3] H. Dombrowski 2023 JINST 18 P07005, doi:10.1088/1748-0221/18/07/P07005