Abstract ID: {to be filled by the organizers}

Design and Development of a High Voltage Pulsed Power Driver for a Tubular Inertial Electrostatic Confinement Fusion Device

Primary author: Sanjib Kalita*1,2

Co-author: Nishant Bharali2, S. R. Mohanty2,3

Presenter: Sanjib Kalita, Nishant Bharali

Track Classification: Track 02

1Department of Electronics and Communication Engineering, Assam Don Bosco University, Guwahati, Assam, India.

2Centre of Plasma Physics – Institute for Plasma Research, Sonapur, Assam, India.

3Homi Bhaba National Institute, Anushaktinagar, Mumbai, Maharashtra, India.

*Corresponding Author: sanjib.kalita@dbuniversity.ac.in

Abstract: Artificial neutron sources are used in many research and industrial fields, and among them, Inertial Electrostatic Confinement (IEC) fusion devices are well known for their small size, flexible operation, and wide range of applications. These devices can produce neutrons in both continuous and pulsed modes, which makes them suitable for future space power systems, making medical isotopes, detecting landmines, treating cancer with boron neutron capture therapy, and creating plasma jets for spacecraft. In this work, a high-voltage Pulsed Power Driver (PPD) was developed at the Centre of Plasma Physics–Institute for Plasma Research (CPP-IPR) to generate pulsed neutrons from a tubular IEC fusion device. The system includes a 0.04 μF, 100 kV capacitor, a spark-gap high-voltage switch, a power supply, resistors, diodes, and a trigger unit. Experimental results recorded with high voltage and current probes demonstrated a peak current of approximately 23.5 A at -35 kV input voltage and a peak power dissipation of around 800 kW, marking significant progress in operating tubular IEC fusion devices in pulsed mode.

References:

- N. Buzarbaruah, N.J. Dutta, D. Borgohain, S.R. Mohanty, H. Bailung, Phys. Lett. A 381, 2391 (2017).

- D. Bhattacharjee, N. Buzarbaruah, S.R. Mohanty, S. Adhikari, Kinetic characteristics of ions in an inertial electrostatic confinement device. Phys. Rev. E 102, 063205 (2020).

- N. Buzarbaruah, N.J. Dutta, J.K. Bhardwaz, S.R. Mohanty, Design of a linear neutron source. Fusion Eng. Des. 90, 97 (2015).

- D. Bhattacharjee, D. Jigdung, N. Buzarbaruah, S.R. Mohanty, H. Bailung, Studies on virtual electrode and ion sheath characteristics in a cylindrical inertial electrostatic confinement fusion device. Phys. Plasmas 26, 073514 (2019).

- S. Kalita, D. Bhattacharjee, & S. R. Mohanty, Development of a compact pulse power driver for operation of table-top fusion device. Eur. Phys. J. D 76, 21 (2022).