Abstract ID: {to be filled by the organizers}

# A Bio-inspired Dual-Parameter Sensor-Based Microstrip Patch Antenna for Soil Moisture and Salinity Detection

Primary author: Muhim Kuli*1

Co-author: Kumaresh Sarmah2, and Bidyut Kumar Baro3

Presenter: Muhim Kuli

Track Classification: Track 02

1,2,3Department of Electronics and Communication Technology, Gauhati University

*Corresponding Author: kulimuhim@gmail.com

Abstract: This paper presents a bio-inspired slotted microstrip patch antenna (MPA) designed as a dual-parameter sensor for monitoring soil moisture and water salinity. The antenna employs a radiating patch patterned with spider-web-like and circular cross-shaped slots, which enhances its sensitivity to variations in the dielectric properties of materials placed in its near field. To enable contactless and non-destructive measurements, a hollow polydimethylsiloxane (PDMS) block is integrated above the patch and functions as a microfluidic chamber for sample analysis. The antenna’s performance was investigated using full-wave electromagnetic simulations in Ansys HFSS under two sensing conditions. For soil moisture, dry soil samples were gradually added with fresh water, increasing the volumetric water content (VWC) from 0% to 80%. In the second case, fresh water was mixed with salt concentrations ranging from 0.5% to 10% (w/v) to evaluate salinity response. The simulation results demonstrated strong sensing capabilities in both scenarios. For soil moisture detection, the antenna exhibited a consistent downward shift in resonant frequency as VWC increased, moving from 5.6 GHz in dry soil (0% moisture content) to 5.19 GHz at the 80% moisture content. For salinity detection, the reflection coefficient (S11) showed clear dependence on ion concentration, with the magnitude increasing from –17.48 dB at low salinity (0.5%) to –16.79 dB at high salinity (10%). This trend was attributed to conductivity-driven impedance mismatching in the water-salt mixtures. By associating frequency changes with soil moisture and S11 variations with salinity, the proposed antenna offers a compact and non-invasive solution for dual-parameter environmental sensing. Its bio-inspired slot structure and microfluidic chamber integration help to ensure high sensitivity and practical use. This novel design demonstrates the antenna's promise for low-cost and efficient monitoring systems in agriculture, irrigation management, and water quality testing.

Keywords: Bio-inspired, dual-parameter sensing, soil moisture and water salinity.

References:

[1] J. Hussein, S. Ahmed, I. Andreea Mocanu, and A. Abbood Al-Behadili. "Microstrip Patch Sensor for Characterizing Saline Solution Based on Complimentary Split-Ring Resonators (SC-SRRs)." Sensors 25, no. 7 (2025): 2319.

[2] K. Warakorn, J. Nualkham, C. Summatta, and S. Sonasang. "Measurement of soil moisture using microwave sensors based on BSF coupled lines." Engineering Proceedings 58, no. 1 (2023): 110.

[3] L. Kibae, A. Hassan, C. Hyun Lee, and J. Bae. "Microstrip patch sensor for salinity determination." Sensors 17, no. 12 (2017): 2941.