Abstract ID: 71

Toward Affordable Multispectral Methods for Optical Glucose Detection Using Sugar Solutions and Blood-Like Tissue Phantoms

Primary author: Dipankar Chetia1*, Nikunja Ranjan Borah1

Presenter: Dipankar Chetia, Nikunja Ranjan Borah

Track Classification: Track 02

1 Optoelectronics and Photonics Laboratory, Department of Physics, North Lakhimpur University

*Corresponding Author: dc@nlc.ac.in

Abstract: Accurately measuring blood glucose levels without drawing blood remains difficult because glucose has weak optical absorption and biological tissue introduces strong background interference. In this work, we explore whether a compact, low-cost multispectral sensor (SparkFun Triad AS7265x, 410–940 nm) can detect glucose-related optical changes. Sugar solutions ranging from 5–35% w/v were measured at several sensor–sample distances, with an 8 mm spacing yielding the most consistent spectral data. After normalization, Partial Least Squares (PLS) regression produced excellent calibration performance (R² ≈ 1.00; RMSE ≈ 0.10%), demonstrating sensitivity to concentration-dependent changes stemming from scattering and refractive index variations.

To better mimic physiological conditions, glucose was added to tissue-like optical phantoms composed of Intralipid and India ink. Preliminary measurements show subtle yet detectable spectral trends in the visible to near-infrared region. Overall, these results suggest that pairing low-cost multispectral sensing with multivariate analysis can provide coarse glucose estimates and represents a promising step toward portable, non-invasive monitoring technologies.

Keywords: multispectral sensing, glucose detection, optical tissue phantom, PLS regression, non-invasive monitoring